### Blue Tooth 2” Thermal Printer G-8BT2:

| APPLICATIONS THAT eMPOWER |  |
| --- | --- |

### Overview:

G-8BT2  is a mobile handheld blue tooth printer can Integrate with Non OS based devices,OS based devices like android, iOS  and Windows etc.  The simple user friendly design allows you to operate with easy and light weight and handy. Inbuilt  2 inch thermal graphic printer enables you to print receipt for the transaction. Collected data can be uploaded to a PC via USB/RS-232 serial interface and  Blue tooth wireless interface.

Bluetooth 4.0

ARM 32 bit Processor

RS232/USB

2” Thermal Printer

USB & RS232 Ports                         Battery Compartment              	       Bill Printing

### Features:

- Bluetooth printer uses58mmFujitsu FTP-628MCL103#70, which have excellent printing quality with a  speed of 75mm/sec.

- Designed for a wide variety of industries that require a cost-effective solution.

- Interfaces: Bluetooth ,USB  and RS232.

- Visual LED indication for paper out, Printer cover open, head temperature abnormality Blue tooth Status

- Supports Total 4 fonts. Can be customizable.

- Supports  Image and Barcode printing

- Bluetooth Signal Transmission Distance 15M/ outdoor 10M / indoor.

- Supports Low power consumption through dynamic power management mode.

- Provides the highest compatibility with the most of OS available in market.

| ### specification: |  |
| --- | --- |
| Model | G-8BT2 |
| Processor | 32 bit High speed CPU @ 50 MHz |
| Memory | 64KB Flash memory, 32KB SRAM, 4MB data flash |
| Printer | 2-inch Thermal Graphic Printer |
| Print method | Line thermal printer |
| Paper width | 57mm |
| Print width | 48mm |
| Weight ( g) | 235 (not including the paper roll) |
| Max diameter of paper roll | 40mm |
| Dot per lie | 384 dots/line |
| Internal characters | ASCII CH(8x16,9x17,9x24,12×24); |
| Print speed | 50~80mm/s |
| Humidity | 95% RH non condensing |
| Life cycle time | 50km(printing density under 12.5%)/100 million pulses |
| Text and graphic Support | English, Logo, image , barcode BARCODE128 |
| Dot pitch (resolution) | 203DPI/8 dots/mm |
| interface | Bluetooth, Mini USB Optional Wi-Fi, GSM, GPRS and Low power Radio |
| Power | 2200mAh 7.4V rechargeable Lithium battery |
| Recharge adapter | Stand by recharging / AC adapter |
| Self-check | Without paper / cover open |
| Paper feeding | Easy paper loading |
| commands | ESC/POS compatible with order sets |
| Paper cut | By hands |
| operate setting | Temperature -10°C~50°C ; Humidity 20%~85% |
| Recharge Setting | Temperature 5°C~40°C; Humidity 20%~85% |
| Storage Setting | Temperature -20°C~70°C ; Humidity 5%~95% |
| Standard accessories | Paper roll ,AC adapter, USB cable |
| Optional accessories | Serial interface cable/leather coat/CD |

### Functional Specifications:

- Test Print Function

| Item | Description |  |
| --- | --- | --- |
|  | After pressing “TEST” Switch the following contents will be print: |  |
|  |  | - GVM Electronic Systems |
|  |  | - Printer Driver card |
|  | . | - Test Print |
|  | . | - Supports four fonts |
|  | . | - Font 1 |
|  | . | - 0123456789 |
|  | . | - abcdefghijklmnopqrstuvwxyz |
|  | . | - ABCDEFGHIJKLMNOPQRSTUVWXYZ |
|  | . | - Font 2 |
|  |  | - 0123456789 |
| Test Print | . | - abcdefghijklmnopqrstuvwxyz |
| Function | . | - ABCDEFGHIJKLMNOPQRSTUVWX |
|  |  | - Font 3 |
|  | . | - 0123456789 |
|  | . | - abcdefghijklmnopqrstuvwxyz |
|  | . | - ABCDEFGHIJKLMNOPQRSTUVWX |
|  | . | - Font 4 |
|  | . | - 0123456789 |
|  | . | - abcdefghij… |
|  | . | - ABCDEFGHIJ… |
|  | . | - Image print support |
|  |  | - “Image” |
|  | . | - Barcode print support |
|  | . | - “Barcode” |
|  | . | - Barcode Number 5512345 |
|  | . | - Test Print completed |

GVM Electronic Systems.

| Registered office: 502, Sri sai land mark, Block-A Street No-8,Habsiguda Hyderabad - 500007, Telangana (INDIA). Website : www.gvmgroup.co.in |  | R&D Center: 1-7-9/3, Level 1 Harihara Estates, Opp ISI, Street No. 8,Habsiguda, Hyderabad - 500007 Telangana (INDIA). Tel : 040 – 40073218. |
| --- | --- | --- |